DETAILED MODULE DESIGN

DFD • Flow Chart • UML Diagram

Project: AI Model to Detect Dryness in Leaves / Agricultural Produce

| Document detail | Description |
| --- | --- |
| Assigned module | AI-Based Dryness Detection and Sensor/Image Data Fusion |
| Team member | Uthejitha Yellampalli |
| Module type | Software / AI module integrated with camera and sensors |
| Purpose | Describe data flow, processing sequence, and software interactions using three diagrams. |

# 1. Module overview

The AI-Based Dryness Detection module receives an image of a leaf or agricultural produce together with available sensor readings and sample metadata. It checks whether the inputs are usable, preprocesses the image and sensor data, runs a trained and validated model or calibrated estimation method, and produces a dryness class such as WET, PARTIALLY DRY or DRY. If supported by validated training data, it may also produce an estimated moisture percentage. The module sends the result and relevant metadata to storage and the dashboard.

Important: an RGB image or environmental temperature/humidity reading alone is not a direct moisture measurement. The model must be calibrated and evaluated against reference moisture-meter readings for the specific target material.

## Module inputs and outputs

| Type | Data | Purpose |
| --- | --- | --- |
| Input | RGB image + sample/batch ID + timestamp | Identify the sample and provide visual features. |
| Input | NIR reading, if available | Provide additional moisture-related signal after calibration. |
| Input | Temperature and relative humidity | Describe drying conditions. |
| Input | Optional sample weight | Track mass loss over time. |
| Reference data | Physical moisture-meter reading | Label training data and validate predictions. |
| Output | Dryness class | WET / PARTIALLY DRY / DRY using validated thresholds. |
| Output | Estimated moisture percentage, if validated | Quantitative prediction with appropriate error reporting. |
| Output | Input-quality flag, timestamp, model version | Support traceability and alert users to unreliable inputs. |

# 2. Data Flow Diagram (DFD)

A DFD shows where data comes from, how the module transforms it, where it is stored, and who receives the result. The diagram below shows a Level 1 view of the dryness-detection module.

## 2.1 DFD Level 1

External entities:
 [E1 Operator]     [E2 Camera / Sensors]     [E3 Reference Moisture Meter]

                 measurement request / sample ID
 [E1 Operator] --------------------------------------------+
                                                            v
 [E2 Camera / Sensors] -- image + sensor readings --> (P1 Acquire & Validate)
                                                            |
                                         invalid input ----+----> error / recapture message --> [E1 Operator]
                                                            |
                                                     valid data
                                                            v
                                                  (P2 Preprocess & Align)
                                                            |
                                                     model-ready data
                                                            v
                                                   (P3 Predict Dryness)
                                                            |
                                         class / estimate / quality flag
                                                            v
                                                   (P4 Format Result)
                                                            |
                                                            +------> [D1 Results Database]
                                                            |         prediction, metadata,
                                                            |         model version, timestamp
                                                            v
                                                    [E1 Operator / Dashboard]
                                                            |
                                                   displayed result / trend

 [E3 Reference Moisture Meter] -- reference moisture label --> [D2 Labeled Dataset]
                                                                    |
                                                    training / evaluation data
                                                                    v
                                                             (P3 Model training/
                                                              calibration - offline)

## 2.2 DFD components and descriptions

| DFD element | Description |
| --- | --- |
| E1 – Operator | Starts a measurement, selects a batch/sample and reviews the result. |
| E2 – Camera/Sensors | Supplies the image and available NIR, temperature, humidity or weight readings. |
| P1 – Acquire & Validate | Checks required fields, sample ID, timestamp, image readability and sensor ranges. |
| P2 – Preprocess & Align | Standardizes the image and associates sensor values with the same sample/time window. |
| P3 – Predict Dryness | Runs the trained model or calibrated estimator to produce a dryness result. |
| P4 – Format Result | Adds class label, estimate if supported, input-quality flag and model version. |
| D1 – Results Database | Stores predictions and metadata for history, evaluation and trend charts. |
| D2 – Labeled Dataset | Stores input records paired with reference moisture readings for training and testing. |

## 2.3 Data stores and data-flow fields

| Record | Suggested fields |
| --- | --- |
| Sample record | sample_id, batch_id, captured_at, image_path, sensor_id |
| Sensor record | temperature_c, relative_humidity_pct, nir_value, weight_g, reading_time |
| Prediction record | dryness_class, estimated_moisture_pct (if validated), quality_flag, model_version, predicted_at |
| Reference label | reference_moisture_pct, measurement_method, measured_at, sample_id |

# 3. Flow Chart of the AI module

The flow chart describes the operational sequence from a user measurement request to the displayed result, including the decision branch for invalid input.

## 3.1 Main processing flow

+------------------+
          |       START      |
          +--------+---------+
                   |
          +--------v---------+
          | Select batch and |
          | sample ID        |
          +--------+---------+
                   |
          +--------v---------+
          | Capture image and|
          | read sensors     |
          +--------+---------+
                   |
          +--------v---------+
          | Validate image,  |
          | readings, ID/time|
          +--------+---------+
                   |
             +-----v------+
             | Inputs     |
             | valid?     |
             +--+------+-+
              No|      |Yes
                |      |
      +---------v--+   +v------------------+
      | Flag error |   | Preprocess image  |
      | and request|   | and align sensor  |
      | recapture  |   | values            |
      +------+-----+   +---------+---------+
             |                   |
             +--> Capture again  |
                                 v
                       +---------+---------+
                       | Run trained model |
                       | / calibrated logic|
                       +---------+---------+
                                 |
                       +---------v---------+
                       | Generate class and|
                       | estimate if valid |
                       +---------+---------+
                                 |
                       +---------v---------+
                       | Save result and   |
                       | metadata          |
                       +---------+---------+
                                 |
                       +---------v---------+
                       | Display status and|
                       | drying trend      |
                       +---------+---------+
                                 |
                       +---------v---------+
                       | More measurements?|
                       +----+----------+---+
                           Yes         No
                            |           |
                            +--> Capture v
                                     +--+--+
                                     | END |
                                     +-----+

## 3.2 Flow chart decisions and error handling

| Decision / condition | Action |
| --- | --- |
| Image missing, blurred or unreadable | Flag image-quality error and ask for recapture. |
| Sensor reading missing or outside allowed range | Mark the reading invalid; continue only if the model supports missing inputs, otherwise request a new reading. |
| No validated model available | Do not present a prediction as reliable; label output as prototype/test mode. |
| Prediction generated | Attach sample ID, timestamp, model version and input-quality status before saving. |
| New measurement requested | Repeat acquisition and processing for the next scheduled interval. |

# 4. UML diagrams

UML (Unified Modeling Language) diagrams describe the system structure and interactions. This document includes a use-case view, a sequence diagram, and a simple class diagram for the assigned software module.

## 4.1 UML Use-Case Diagram

+---------------------------------------------+
                    |      AI DRYNESS DETECTION SYSTEM             |
                    |                                             |
Operator ---------->| (Select batch / sample)                     |
Operator ---------->| (Request measurement)                        |
Operator <----------| (View dryness result and trend)              |
Operator ---------->| (Review input warning)                       |
                    |                                             |
Camera / Sensors -->| (Provide image and sensor data)              |
                    |                                             |
Reference Meter --->| (Provide reference moisture label)           |
                    |                                             |
                    | (Validate and preprocess input)              |
                    | (Run dryness model)                           |
                    | (Store prediction and metadata)              |
                    +---------------------------------------------+

Actors: Operator, Camera/Sensors and Reference Measurement Technician. Main use cases: select sample, request a measurement, provide data, validate inputs, predict dryness, store results and view history.

## 4.2 UML Sequence Diagram

Operator       Capture Service     AI Module       Database       Dashboard
   |                  |                 |               |               |
   |--request test--->|                 |               |               |
   |                  |--image + data-->|               |               |
   |                  |                 |--validate     |               |
   |                  |                 |--preprocess   |               |
   |                  |                 |--predict      |               |
   |                  |                 |--save result->|               |
   |                  |                 |<--save ack----|               |
   |                  |                 |--result---------------------->|
   |<------------------------- display result / warning ----------------|
   |                  |                 |               |               |

Sequence explanation: the operator requests a measurement; the capture service supplies an image and readings; the AI module validates and preprocesses them, runs inference, saves the prediction and returns the result for display.

## 4.3 UML Class Diagram (conceptual)

+-----------------------+        +-----------------------+
| SampleRecord          |        | SensorReading         |
+-----------------------+        +-----------------------+
| sample_id             | 1    * | timestamp             |
| batch_id              |--------| temperature_c         |
| captured_at           |        | humidity_pct          |
| image_path            |        | nir_value             |
+-----------------------+        | weight_g (optional)   |
                                 +-----------------------+

+-----------------------+        +-----------------------+
| DrynessDetector       |------->| PredictionResult      |
+-----------------------+        +-----------------------+
| model_version         |        | sample_id             |
| validate_input()      |        | dryness_class         |
| preprocess()          |        | moisture_pct optional |
| predict()             |        | quality_flag          |
+-----------------------+        | predicted_at          |
                                 +-----------------------+
                                            |
                                            v
                                 +-----------------------+
                                 | ResultRepository      |
                                 +-----------------------+
                                 | save_result()         |
                                 | get_history()         |
                                 +-----------------------+

| Class | Responsibility |
| --- | --- |
| SampleRecord | Stores sample and batch identifiers and the captured image reference. |
| SensorReading | Stores timestamped sensor values associated with a sample. |
| DrynessDetector | Validates inputs, preprocesses features and calls the trained model. |
| PredictionResult | Represents the predicted class, optional moisture estimate and quality metadata. |
| ResultRepository | Persists prediction records and retrieves history for the dashboard. |

# 5. Integration and module interfaces

| Interface | Example payload / fields | Validation requirement |
| --- | --- | --- |
| Capture → AI module | sample_id, batch_id, timestamp, image_path, sensor readings | Required fields present; correct sample and time association. |
| AI module → database | sample_id, predicted_at, dryness_class, optional moisture_pct, quality_flag, model_version | Store only complete, traceable prediction records. |
| Database → dashboard | latest result, trend history, warnings | Display units, timestamp and warnings clearly. |
| Reference meter → training data | sample_id, reference_moisture_pct, method, measured_at | Reference value must correspond to the same sample. |

# 6. Suggested development and testing checklist

Define the input record format and sample-ID convention.

Capture representative images and sensor values at multiple drying stages.

Record reference moisture-meter readings for the same samples.

Implement input validation and image preprocessing.

Train or configure a baseline model and keep training/test data separate.

Evaluate moisture estimates with MAE/RMSE or classes with precision, recall, F1-score and a confusion matrix.

Test invalid images, missing readings and repeated timestamps.

Verify predictions are saved and the dashboard displays the right sample and timestamp.

# Conclusion

The DFD explains the data sources, processing steps and data stores; the flow chart explains the operational logic and error branches; and the UML diagrams describe actors, interactions and the software entities. Together, these diagrams provide a detailed design for the AI-Based Dryness Detection and Sensor/Image Data Fusion module.